Self-potential Tomography Preconditioned by Particle Swarm Optimization—Application to Monitoring Hyporheic exchange in a Bedrock River

Scott J. Ikard, U.S. Geological Survey, Austin, Texas

Kenneth C. Carroll, New Mexico State University, Las Cruces, New Mexico

Dale F. Rucker, hydroGEOPHYSICS, Inc., Tucson, Arizona

Scott C. Brooks, Oak Ridge National Laboratory, Oak Ridge, Tennessee

# Abstract

A self-potential (SP) data inversion algorithm was tested on synthetic geoelectrical models of electrical-potential profile data attributed to single and multiple polarized electrical sources and then applied to SP-monitoring field data measured on the floodplain of East Fork Poplar Creek, Oak Ridge, Tennessee, to image electrical sources in a preferential flow path into the flood plain from the bedrock-lined riverbed. The tested algorithm combined stochastic source localization by particle-swarm-optimization (PSO) of an analytical forward model of electrical sources characterized by spherical, cylindrical, and planar geometries with source tomography by regularized weighted least-squares minimization of a quadratic objective function. Prior information is incorporated by linear subspace inversion methodologies in addition to preconditioning of the kernel matrix by properties of stochastic sources predetermined by PSO. Synthetic analytical-model data were contaminated with variable percentages of random noise to evaluate the algorithm performance in the presence of data noise. Results indicated that true parameters of single-source models were inverted and approximated with low residual error, whereas inversion of synthetic analytical-model data representing multiple electrical sources accurately approximated the geospatial coordinates of the sources but miscalculated some parameter values because of the non-uniqueness of the inverse-model solution. When applied to SP-monitoring field data, the algorithm successfully imaged electrical sources within a known fault that intersects the bedrock riverbed and flood plain of East Fork Poplar Creek and depicts dynamic electrical conditions within the fault attributed to hyporheic exchange driven by surface-water losses and gains.