CORRELATION OF EM38 AND VERIS APPARENT ELECTRICAL CONDUCTIVITY TO SOIL PROPERTIES OF AGRICULTURAL FIELDS

Leti T. Wodajo1, Martin A. Locke2, Robert Wade Steinriede, Jr.2 Md Abdus Samad1, and Craig J. Hickey1

1. National Center for Physical Acoustics, The University of Mississippi, University MS, USA

2. National Sedimentation Laboratory, USDA Agricultural Research Service, Oxford, MS, USA

Abstract

In agricultural fields, collecting and processing soil samples provide the most accurate data on soil properties. However, they are usually collected in a coarse grid, time-consuming to process, and could miss variabilities between sampling locations. On the other hand, geophysical measurements such as electromagnetic induction (EMI) surveys can be done at a much higher sampling rate and provide high-resolution spatial maps with detailed field variability. A better understanding of the correlation between geophysical and soil properties is required for an improved interpretation of geophysical results. In this study, apparent electrical conductivity (ECa) measurements were collected on an agricultural field using a Geonics EM38 and a Veris U series survey instruments. The study site in Sunflower, MS, is part of USDA’s Conservation Effects Assessment Project (CEAP) network, where soil physical properties were evaluated over multiple years. Correlation analysis was performed between ECa obtained from both survey methods and USDA’s soil properties data (clay, water content, and soil EC). A comparison of EM38 and Veris ECa maps showed that, while there are differences in the magnitude of the measured ECa values, the spatial distribution of ECa in the field is quite similar. ECa measurements from EM38 and Veris showed good correlations with measured clay content. However, ECa's correlation with water content was low, probably because the soil water content and the survey assessments were done at different times and in different field conditions. Overall, this study showed that the application of rapid geophysical methods and establishing correlations with soil properties could lead to better soil and crop management decisions through the identification of farm field variability, improved soil sampling as opposed to grid-based soil sampling, and generating spatial variability maps of parameters relevant to plant growth (e.g., clay content, soil salinity, soil texture, soil moisture).

[This work was supported by the U.S. Department of Agriculture under Non-Assistance Cooperative Agreement 58-6060-6-009. Any opinions, findings, conclusions, or recommendations expressed in this publication are those of the author(s) and do not necessarily reflect the views of the U. S. Department of Agriculture.]